Using Respirators

Only wear a clean respirator. 
Do not re-use disposable products.

Always follow the manufacturer’s instructions.

How to put on a respirator

Wash your hands before putting on your respirator.

Long hair must be tied back. Face must be clean-shaven (no facial hair).

- Place the mask over the nose and mouth.

- Pull the top strap over your head and position it at the back of your head, with the strap above your ears.

- Pull the bottom strap over your head and position it at the back of your neck, with the strap below your ears.

- Shape the nose clip to the bridge of your nose.

- Make sure the mask fits snugly around the face. Perform a seal check.

Perform a seal check

- Cup both hands over the front respirator.

- If wearing a non-valved respirator – exhale sharply.

If wearing a valved respirator – inhale sharply.

- Air should not leak around the mask. If it does, mold the nose clip again and reposition the straps.

How to remove a mask

The front of the mask or respirator may be contaminated – DO NOT TOUCH IT.

- Wash your hands.

- Grasp the mask ONLY by the straps. Pull the strap at the back of your neck over your head, then pull off the top strap.

- Dispose of the mask.

- Wash your hands.